INTRODUCTION

Our Ref:

To determine the effeciency of the dispersant FINASOL OSR51 using the AMSA Reference oil.

TESTS

The MacKay apparatus was set to an air and seawater temperature of 20 ºC with a moderate wave action height of 3.5 cm.

A 1:20 ratio is aimed at for the dispersant/oil mixture.

After a 10 minute period of mixing in the MacKay test chamber, a subsample is taken (A cut). The wind is then turned off. After a further 5 minutes, another subsample is taken (Q cut) and samples are subsequently extracted with Methylene Chloride and analysed on a double beam spectrophotometer.

RESULTS

A good dispersant will immediately break up the oil to really fine droplets and will produce a coffee coloured appearance in the Mackay test chamber. The finer the droplets the longer they will stay in suspension and therefore the % recovery of oil in the “Q cut” (quiescence) ultimately determines this.

Three replicates are performed on each dispersant. This gives a good indication of how a dispesant performs. Ideally, to validate any method, seven replicates should be performed.

FINASOL OSR 51

The aim of the test is to try and get a 20:1 ratio as accurately as possible using the known density of oil and dispersant. Because these procedure are determined by weight, sometimes this ratio will vary slightly due to residual amount of oil or dispersant left inside the syringe after dispensing into the MacKay apparatus.

Finasol OSR 51 performed well, instantly breaking up the oil into fine droplets which remained in suspension well past the 15 minute mark.

Results reproduced well with the exception of the first 5Q.  which was not used in the Mean and STDEV.